SUMMARY Great Broughton Speed Survey 2022

Data recorded 03/02/2022 10:00:00 through 14/08/2022 14:59:59

Introduction

In 2020 Great and Little Broughton Parish Council purchased 2 Vehicle Activated Speed (VAS) warning signs which are located at the North and South ends of the village. The speed limit throughout the village is 30mph and the VAS signs activate at 33mph (speed limit + 10%).

Data for the preceding 6 months was downloaded on 14.8.22 and analysed using Houston Radar software to determine the 85th Percentile (this is the speed at or below which 85% of all vehicles are observed to travel past the nominated point, commonly used as it represents “a reasonable driver” - 85% = mean 50% + 1 SD 34.1% assuming a normal distribution).

Overview of Great Broughton Traffic data

- GB South (incoming traffic from Chopgate)

Speed limit 30mph- 15% vehicles above this

Vehicle activated speed sign 33mph

Average speed 26mph of all traffic, average speed of those exceeding limit 33mph

Max Speed 65 mph on 26/05/2022 06:15:00

85th Percentile Speed = 30mph

Traffic volume counts per hour recorded range from 0 to 195/hour (5-6am Sun 19.6.22 – also high volume 6-7am and 7-8am same day) and 190/hour (11am-12 Sun 20.3.22).

Total traffic counted 203609 with average daily volume estimated at 1059.

Busiest hour is usually 11am-12, with traffic building up steadily to this peak through morning and tailing off from early afternoon – a similar pattern observed on both weekdays and weekend days.

Similar speed pattern is observed at weekends.

- GB North (incoming traffic from Stokesley)

Speed limit 30mph- 24% vehicles above this

Vehicle activated speed sign 33mph

Average speed 27.5mph of all traffic, average speed of those exceeding limit 38mph

Max Speed 60 MPH on 22/03/2022 19:45:00

85th Percentile Speed = 32mph

Traffic volume counts per hour recorded range from 0 to 240/hour (11am-12 2.6.22).

Total traffic counted 335708 with average daily traffic 1748.

On weekdays the busiest hour is usually 5-6pm though traffic remains high from 12-2pm and from 3-7pm. On weekends the busiest hour is usually 12-1 though traffic remains high from 10am-5pm.

Observations

a. Speed

Average speed of drivers at both ends of village is below speed limit.

85 percentile speed average is 30mph at GB South and 32mph at GB North meaning that a significant minority of drivers are above speed limit entering village.

Lower average 85percentile speeds tend to be seen during daytime and higher average 85percentile speeds seen during evening and overnight.

There is no clear difference observed in speed patterns between weekdays and weekends.

b. Volume

GB South has average daily volume estimated at 1059. Busiest hour is usually 11am-12, with traffic building up steadily to this peak through morning from early hours and tailing off from early afternoon – a similar pattern observed on both weekdays and weekend days.

GB North has average daily traffic volume estimated at 1748. On weekdays the busiest hour is usually 5-6pm; traffic remains high throughout working hours with peaks from 12-2pm and from 3-7pm. On weekends the busiest hour is usually 12-1.

Attachments:

- Bar chart vehicle count vs speed GB North Data for 6m to August 22

- Bar chart vehicle count vs speed GB South Data for 6m to August 22

- Percentile chart for speed Great Broughton North Data for 6m to Aug 22

- Percentile chart for speed Great Broughton South Data for 6m to Aug 22

CW, 14.10.22